FILENAME: 8255_PRG.DOC   UPDATED 10-November-93 by Don McKenzie.

To get the maximum use out of the 8255 PPI (Programmable Peripheral

Interface), you really need to get the data sheets from a manufacturer, or

out of a Technical book that covers this device.

I have decided to cover here the simplest and possibly most used mode of the

8255 when it is pressed into service for a micro controller.

There are three modes of operation:

Mode (0) Basic Input/Output which I will cover.

Mode (1) Strobed Input/Output which is used on PBUFF. Portions of port C are

used as handshaking lines for ports A and B. Either A or B can be

programmed as Input or Output.

Mode (2) Strobed Bidirectional Bus Input/Output. I have never used it, but I

am sure it could be put to excellent use.

_______________________________________________________________

Mode (0) Basic Input/Output.

In this mode, simple input and output operations for each of the three ports

are provided. No "handshaking" is required. Data is simply written to or

read from a specified port.

Mode 0 Port Definition Chart:

Port C lower bits are PC0-PC3, 4 bits.

Port C upper bits are PC4-PC7, 4 bits.

��������������������������������������������������������������Ŀ

�          �   CONTROL WORD BITS   �  GROUP A    �  GROUP B    �

�          ���������������������������������������������������Ĵ

�          �  �  �  �  �  �  �  �  �      �PORT C�      �PORT C�

�Dec Hex No�D7�D6�D5�D4�D3�D2�D1�D0�PORT A�UPPER �PORT B�LOWER �

��������������������������������������������������������������Ĵ

�128�80 �00�1 �0 �0 �0 �0 �0 �0 �0 �OUTPUT�OUTPUT�OUTPUT�OUTPUT�

��������������������������������������������������������������Ĵ

�129�81 �01�1 �0 �0 �0 �0 �0 �0 �1 �OUTPUT�OUTPUT�OUTPUT�INPUT �

��������������������������������������������������������������Ĵ

�130�82 �02�1 �0 �0 �0 �0 �0 �1 �0 �OUTPUT�OUTPUT�INPUT �OUTPUT�

��������������������������������������������������������������Ĵ

�131�83 �03�1 �0 �0 �0 �0 �0 �1 �1 �OUTPUT�OUTPUT�INPUT �INPUT �

��������������������������������������������������������������Ĵ

�136�88 �04�1 �0 �0 �0 �1 �0 �0 �0 �OUTPUT�INPUT �OUTPUT�OUTPUT�

��������������������������������������������������������������Ĵ

�137�89 �05�1 �0 �0 �0 �1 �0 �0 �1 �OUTPUT�INPUT �OUTPUT�INPUT �

��������������������������������������������������������������Ĵ

�138�8A �06�1 �0 �0 �0 �1 �0 �1 �0 �OUTPUT�INPUT �INPUT �OUTPUT�

��������������������������������������������������������������Ĵ

�139�8B �07�1 �0 �0 �0 �1 �0 �1 �1 �OUTPUT�INPUT �INPUT �INPUT �

��������������������������������������������������������������Ĵ

�144�90 �08�1 �0 �0 �1 �0 �0 �0 �0 �INPUT �OUTPUT�OUTPUT�OUTPUT�

��������������������������������������������������������������Ĵ

�145�91 �09�1 �0 �0 �1 �0 �0 �0 �1 �INPUT �OUTPUT�OUTPUT�INPUT �

��������������������������������������������������������������Ĵ

�146�92 �10�1 �0 �0 �1 �0 �0 �1 �0 �INPUT �OUTPUT�INPUT �OUTPUT�

��������������������������������������������������������������Ĵ

�147�93 �11�1 �0 �0 �1 �0 �0 �1 �1 �INPUT �OUTPUT�INPUT �INPUT �

��������������������������������������������������������������Ĵ

�152�98 �12�1 �0 �0 �1 �1 �0 �0 �0 �INPUT �INPUT �OUTPUT�OUTPUT�

��������������������������������������������������������������Ĵ

�153�99 �13�1 �0 �0 �1 �1 �0 �0 �1 �INPUT �INPUT �OUTPUT�INPUT �

��������������������������������������������������������������Ĵ

�154�9A �14�1 �0 �0 �1 �1 �0 �1 �0 �INPUT �INPUT �INPUT �OUTPUT�

��������������������������������������������������������������Ĵ

�155�9B �15�1 �0 �0 �1 �1 �0 �1 �1 �INPUT �INPUT �INPUT �INPUT �

����������������������������������������������������������������